# Experiment 8

Student Name: Sahil Kaundal					UID: 21BCS8197

Branch: CSE (Lateral Entry)		                          	Section/Group: 616/A

Semester: 6th								Date of Performance: 03/05/2023

Subject Name: Internet of Things Lab				Subject Code: 20CSP-358

- Aim:

## Interfacing Air Quality Sensor (MQ135), displays data on LCD.

- Objective:

- Learn about LCD in detail.

- Learn about IoT programming.

- Interfacing Air Quality Sensor (MQ135), displays data on LCD.

- Requirements:

- Arduino Uno R3 board

- LCD

- Jumper Wires

- MQ135 Air Quality Sensor.

- Procedure:

About Air Quality Sensor:

MQ-135 sensor belongs to the MQ series that are used to detect different gasses present in the air. The MQ-135 sensor is used to detect gases such as NH3, NOx, alcohol, Benzene, smoke, CO2, etc. steel exoskeleton houses a sensing device within the gas sensor module.

#### Specifications

The table below shows some key specifications of the MQ-135 sensor module:

| Feature | Description |
| --- | --- |
| Operating Voltage | 2.5-5.0V |
| Detecting Concentration | 10ppm-300ppm for NH3 10ppm-1000ppm for Benzene 10ppm-300ppm for Alcohol |
| Load Resistance | Adjustable |
| Heater Resistance | 33Ω ± 5% |
| Heater Consumption | less than 800mW |
| Operating Temperature | -10 to 45°C |

#### Pinout

##### MQ-135 Sensor Pinout

##### This sensor has 4 pins:

##### 5V: Module power supply – 5 V

##### GND: Ground

##### DOUT: Digital output

##### AOUT: Analog output

Circuit

The MQ-135 sensor module consists of four pins namely VCC, GND, DO, and DO. The table below gives a brief description of them.

| Pin | Description |
| --- | --- |
| VCC | Positive power supply pin that powers up the sensor module. |
| GND | Reference potential pin. |
| AO | Analog output pin. It generates a signal proportional to the concentration of gas vapors coming in contact with the sensor. |
| DO | Digital Output pin. It also produces a digital signal whose limit can be set using the in-built potentiometer. |

- Steps/Program:

int sensorValue;

int digitalValue;

void setup()

{

Serial.begin(9600); // sets the serial port to 9600

pinMode(13, OUTPUT);

pinMode(2, INPUT);

}

void loop()

{

sensorValue = analogRead(0); // read analog input pin 0

digitalValue = digitalRead(2);

if (sensorValue > 400)

{

digitalWrite(13, HIGH);

}

else

digitalWrite(13, LOW);

Serial.println(sensorValue, DEC); // prints the value read

Serial.println(digitalValue, DEC);

delay(1000); // wait 100ms for next reading

}

#include "LiquidCrystal.h"

LiquidCrystal lcd(8,7,6,5,4,3);

int GAS_VAL = 0;

void setup()

{

pinMode(A0, INPUT);

Serial.begin(9600);

lcd.begin(16,2);

lcd.setCursor(0,0);

lcd.print("  Experiment 7 ");

}

void loop()

{

GAS_VAL = analogRead(A0);

Serial.println(GAS_VAL);

if (GAS_VAL > 500)

{

lcd.setCursor(0,1);

lcd.print(" GAS   Detected");

}

else

{

lcd.setCursor(0,1);

lcd.print(" GAS Not Detect");

}

delay(10); // Delay a little bit to improve simulation performance

}

- Output:

Learning outcomes (What I have learnt):

- Learn about LCD in detail.

- Learn about IoT programming.

- Interfacing Air Quality Sensor (MQ135), displays data on LCD.